CSC Direct Access (DA)

# Local Area Network Requirements FOR MIS National (DA)

The following requirements apply to the customer managed IP network.

### System Signaling and Media Ports firewall requirements

The following tables list the firewall requirements to support call control (SIP) signaling, feature functionality, management, and voice payload (RTP) for the CLEARSPAN SIP services.

ALL SIP ALG’s MUST be DISABLED in the Firewall and Routers!!!!

#### OUTBOUND FROM CUSTOMER’s REMOTE phones to clearspan service

Port and Protocol requirements may vary for customer specific configurations.

| From Hosted Customer | To Service Provider |  | Destination Port |
| --- | --- | --- | --- |
| Customer Premise | 209.43.79.135, 209.43.79.134, 67.217.172.7, 67.217.172.6, Extended Service Platform (XSP) |  |  |
|  |  |  | 443 / TCP (HTTPS) 80 TCP (HTTP) |
| Customer Premise | 209.43.79.136 and 67.217.172.8 Unified Messaging Server (UM) |  | 993 TCP (Secure IMAP) |
| Customer Premise | 209.43.79.138 and 67.217.172.10 Element Manager Server (EMS) |  | 443 TCP (HTTPS) 80 TCP (HTTP) 123 UDP (NTP) |
| Customer Phone | 209.43.79.142 and 67.217.172.13 SIP Session Manager (SSM) SIP 209.43.79.143 and 67.217.172.14 SIP Session Manager (SSM) RTP |  | 5080/ UDP (SIP) |
|  |  |  | 5080 / TCP (SIP) Mobile Communicator 5083 / TCP Encryption (SIP) |
|  |  |  | 6000-65000 / UDP (RTP) 6000-65000 / UDP (SRTP) |
|  |  |  | 6000-65000 / UDP (t.38) |
| Customer Premise |  | Customer provided DNS Servers | 53 /TCP and UDP (DNS) |

- INBOUND TO customer’s Remote PHONEs from the clearspan service

- Port and Protocol requirements may vary for customer specific configurations.

| From Service Provider | To HVS National Customer | Destination Port |
| --- | --- | --- |
| 209.43.79.135, 209.43.79.134, 67.217.172.7, 67.217.172.6, Extended Service Platform (XSP) | Customer Premise | 443 / TCP (HTTPS) 80 TCP (HTTP) |
| 209.43.79.138 and 67.217.172.10 Element Manager Server (EMS) | Customer Premise | 443 TCP (HTTPS) 80 TCP (HTTP) 123 UDP (NTP) |
| 209.43.79.136 and 67.217.172.8 Unified Messaging Server (UM) | Customer Premise | 993 TCP (Secure IMAP) |
| 209.43.79.142 and 67.217.172.13 SIP Session Manager (SSM) SIP 209.43.79.143 and 67.217.172.14 SIP Session Manager (SSM) RTP | Customer Phone | 5080 / UDP (SIP) |
|  |  | 5080 / TCP (SIP) Mobile Communicator 5083 / TCP Encryption (SIP) |
|  |  | 6000-65000 / UDP (RTP) 6000-65000 / UDP (SRTP) |
|  |  | 6000-65000 / UDP (t.38) |
| Customer provided DNS Servers | Customer Premise | 53 /TCP and UDP (DNS) |

| DHCP Server Services Information |  |
| --- | --- |
| DNS Server #1 | Customer Provided |
| DNS Server #2 | Customer Provided |
| NTP Server #1 | 209.43.79.138 |
| NTP Server #2 | 67.217.172.10 |
| Option 66 | https://pub-xs.sip.clearspancloud.net/dms/clearspan or http://pub-xs.sip.clearspancloud.net/dms/clearspan |